GTS Mirror Conversion

Wiring Diagram M.Y.1980 to 1984

HEATER

BR

MOTOR

YE

MOTOR

MM

MIRROR						  GR

GN

WT           BL            BR               BR            RE         BK		DO NOT

USE THESE

FOUR WIRES

CAR

JUMPER

BK                BK            BR                 WT          BR       GR/GN     DRIVER

BK                BL	       BR		 WT      	  BR       GR/GN    PASSENGER

NOTE: - TO CHANGE HORIZONTAL TO VERTICAL MOVEMENT, SWAP WT & BL  MIRROR WIRES

WITH RE & BK MIRROR WIRES

-TO SWAP LINEAR MOTION OF A  PARTICULAR MOVEMENT, SWAP A PAIR OF MIRROR

WIRES (EX: SWAP WT WITH BL  AND/OR  RE  WITH BK)

- DO NOT USE FOUR OTHER WIRES AS THEY SERVE SEAT/MIRROR MEMORY ONLY

- DO NOT REPLACE WINDOW DIRECTIONAL SWITCH

BL -BLUE    		BK -BLACK    	              GR-GREY	       YE - YELLOW

GN-GREEN                        WT-WHITE                        BR -BROWN               RE - RED

GR/GN - GREY WITH GREEN STRIPE